# Network

- Network core functions

- Forwarding

  - Also called “switching”

  - Local action

  - Like changing streets on a much larger path

  - Uses forwarding table, first column being header value and second value being output link

    - The header would be an ip address

    - Uses header value to send to output link

- Routing

  - Global action:

    - Determine source destination paths taken by packets

  - Routing algorithms

- Package transmission rate: L/R

  - L is bits per packet and R is bits per second rate of transmission

  - Entire packet must arrive at router before it can be transmitted on next link

  - Packet loss occurs when packets arrive into a queue due to the output rate being slower than the input rate and the queue becomes filled.

- Frequency Division Multiplexing

  - EMF divided into narrow frequency bands at a max rate of that narrow band

- Time Division Multiplexing

  - Time divided into slots

- Packet switching vs circuit switching

  - Packet switching is better for “bursty” resource sharing and simpler, no call setup

  - Packet switching can cause delay and loss if there are too many users sending data at the same time

  - Circuit switching has a maximum number of users

    - Everyone gets same share

  - Packet switching uses protocol to ensure everyone gets same share

- IXP

  - Connects different ISP’s

- Packet delay

  - dnodal = dproc +dqueue +dtrans+dprop

  - Nodal determines output links

    - Microseconds

  - Queue delay is large

  - Trans delay is the transmission rate delay

    - L/R  (packet length/transmission rate)

  - Prop delay is the length of physical link

    - d/s (distance divided by speed)

- Packet queueing delay

  - (L*a)/R

    - a= average packet arrival rate

- Throughput

  - bits/time unit at which bits are being sent

    - The weaker transmission rate becomes the end to end throughput

  - Bottleneck link

- Layered internet protocol stack

  - multiple segments:

  - Application

    - Supporting network applications

      - HTTP, IMAP, SMTP, DNS

  - Transport

    - Process-process data transfer

      - TCP, UDP

      - Ex: website runs on a server process

| source port # | destination port # |
| --- | --- |
| other header fields |  |
| application data (payload) |  |

  - Network

    - Routing of datagrams from source to destination

      - IP, routing protocols

  - Link

    - Data transfer between neighboring network elements

      - Ethernet, 802.11 (WiFi), PPP

  - Physical

    - Bits “on the wire”

- Internet History

  - Protocols

    - 1983-TCP/IP protocol

    - 1982-smtp email protocol

    - 1983-DNS define for name to IP address translation

    - 1985-ftp protocol defined

    - 1988-TCP congestion control

    - new networks: CSnet, BITnet, NSFnet,Mininet

  - 1990's-ARPAnet decommissioned

  - 1991-NSF lifts restrictions on commercial use of NSFnet (decommissioned 1995)

  - 1990's:Web: Hypertext, HTML, HTTP, Mosaic (later netscape), commercialization

  - 2000's: instant messaging, P2P  file sharing, network security to forefront,

    - 50 mil host, 100 mil users

    - backbone links running at Gbps

  - 2005-Aggressive deployment of broadband home access (10-100Mbps)

  - 2008-software defined networking (SDN)

  - increasing ubiquity of high-speed wireless access: 4G/5G, WiFi

  - service providers create their own networks

  - Enterprises run their services in "cloud"

  - 2017-rise of smartphones: more mobile than fixed devices on internet

- ISO/OSI reference model

  - 7 layers

    - Application, Presentation, Session, Transport, Network, Link, Physical

  - presentation:

    - allows applications to interpret meaning of data, e.g., encryption, compression, machine-specific conventions

  - session:

    - synchronization, checkpointing, recovery of data exchange

  - Internet stack is the same without these two layers.

### Creating a network app

- Write programs that:

  - run on different end systems

  - communicate over network

  - web server software communicates with browser software

- No need to write software for network-core devices:

  - network-core devices do not run user applications

  - applications on end systems allows for rapid app development, propagation

- Server:

  - always on host

  - permanent IP address

  - often in data centers, for scaling

- Client:

  - contact, communicate with server

  - may be intermittently connected

  - dynamic IP addresses

  - do not communicate directly with each other

  - examples? HTTP, IMAP, FTP

- Peer-peer architecture

  - no always-on server

  - arbitrary end systems directly communicate

  - peers request service from other peers, provide service in return to other peers

    - self scalability-new peers bring new service capacity, as well as demands

- Sockets

  - socket analogous to a door

  - it's on both hosts

- Addressing processes

  - to receive messages process must have identifier

  - host has unique 32-bit IP  address

  - port number identifies the process on host

- An application-layer protocol defines:

  - types of messages exchanged,

    - e.g., request, response

  - message syntax:

    - what fields in messages & how fields are delineated

  - message semantics

    - meaning of information in fields

  - rules

    - when and how processes send and respond to messages

  - open protocols

    - public info in RFCs (Request for comments)

    - allows for interoperability

  - proprietary protocols:

    - skype, zoom

  - some apps require 100% reliable data transfer

- Data integrity - What transport service do they need?

  - some apps require 100% reliable data transfer

  - other apps (e.g., audio/video) can tolerate some loss

- timing

  - some apps (e.g., Internet telephony interactive games)

    - require low delay to be "effective"

- throughput

  - some apps(e.g., multimedia) require minimum amount of throughput to be "effective"

  - other apps ("elastic apps") make use of whatever throughput they get

- security

  - encryption, data integrity

- TCP service

  - reliable data transport

  - flow control: sender won't overwhelm receiver

  - congestion control: throttle sender when network overloaded

  - connection-oriented: setup required between client and server processes

  - doesn't provide:

    - timing,minimum throughput guarantee, security

- UDP service

  - unreliable data transfer

  - doesn't provide:

    - reliability, flow control, timing, throughput guarantee, security, or connection setup

- TLS - Transport Layer Security

  - provided encrypted TCP connections

- Web and HTTP

  - web page consists of objects, each of which can be stored on different Web servers

  - objects can be HTML file, JPEG,  Java applet, audio file, etc.

  - web page consists of base HTML-file which includes several referenced objects, each addressable by a URL, e.g.

- Non-persistent HTTP

  - TCP connection opened

  - at most one object sent over TCP connection

  - TCP connection closed

    - downloading multiple objects requires multiples connections

  - Non-Persistent HTTP response time = 2RTT + file transmission time

- Persistent HTTP

  - TCP connection opened to server

  - multiple objects can be sent over single TCP connection between client and that server

  - TCP connection closed

- www.someSchool.edu/someDepartment/home.index

- cookies

  - used to remember some state between transactions because HTTP protocol doesn't

- Web caches (aka proxy servers)

  - user configures browser to point to a local web cache

  - client to origin server

  - rate to browsers over access link = % requests satisfied at origin / average data access

  - access link utilization = mbps of rate to browers over access link divided by access link speed

  - end to end delay = % satisfied at origin * delay from origin + % requests served by cache * delay at cache

- SMTP sending email protocol

- IMAP: protocol for retrieving

- DNS: Domain Name System

  - many IP's for one name

  - billions of simple records in a database

  - decentralized and secure system

  - billions of users

  - Register name at DNS registrar

- DNS info ex:

  - name: networkutopia.com

  - address mapping: dns1.networkutopia.com

  - IP: 212.212.212.1

  - create authoritative server locally with IP address 212.212.212.1

    - typeA record for www.networkutopia.com

    - type MX record for networkutopia.com

    - type A for email.networkutopia.com, 3.4.5

  - Local DNS servers query to root DNS server queries back with address to .com DNS server which sends IP of the website back to local DNS

- Multimedia: video

  - we use spatial coding for a portion of an image that is the same color

  - we use temporal coding for one frame to the next

    - code difference instead of a whole new frame

  - CBR: constant bit rate

    - MPEG1

  - VBR: variable bit rate

    - MPEG2

    - MPEG4

  - buffering is done to have video frames queued up before the video starts playing

  - DASH: Dynamic Adaptive Streaming over HTTP

    - server divides video file into multiple chunks

    - each chunk encoded at multiple different rates

    - different rate encodings stored in different files

    - files replicated in various CDN nodes

    - manifest file: provides URLs for different chunks

    - client estimates bandwidth

    - consults manifest, requests one chunk at a time depending on bandwidth

- CDN: Content Distribution Networks

  - deploys multiple servers

- UDP Client Python:

  - from socket import *

  - serverName = ‘hostname’

  - serverPort = 12000

  - clientSocket = socket(AF_INET,SOCK_DGRAM)

  - message = raw_input(’Input lowercase sentence:’)

  - clientSocket.sendto(message.encode(),(serverName, serverPort))

  - modifiedMessage, serverAddress =clientSocket.recvfrom(2048)

  - print modifiedMessage.decode()

  - clientSocket.close()

- UDP Server Python

  - from socket import *

  - serverPort = 12000

  - serverSocket = socket(AF_INET, SOCK_DGRAM)

  - serverSocket.bind(('', serverPort))

  - print (“The server is ready to receive”)

  - while True:

    - message, clientAddress = serverSocket.recvfrom(2048)

    - modifiedMessage = message.decode().upper()

    - serverSocket.sendto(modifiedMessage.encode(),clientAddress)

- TCP Client Python

  - from socket import *

  - serverName = ’servername’

  - serverPort = 12000

  - clientSocket = socket(AF_INET, SOCK_STREAM)

  - #^create TCP socket for server, rmv port 12000

  - clientSocket.connect((serverName,serverPort))

  - sentence = raw_input(‘Input lowercase sentence:’)

  - clientSocket.send(sentence.encode())

  - modifiedSentence = clientSocket.recv(1024)

  - print (‘From Server:’, modifiedSentence.decode())

  - clientSocket.close()

- TCP Server Python

  - from socket import *

  - serverPort = 12000

  - serverSocket = socket(AF_INET,SOCK_STREAM)

  - serverSocket.bind((‘’,serverPort))

  - serverSocket.listen(1)

  - print('The server is ready to receive’)

  - while True:

    - connectionSocket, addr = serverSocket.accept()

    - sentence = connectionSocket.recv(1024).decode()

    - capitalizedSentence = sentence.upper()

    - connectionSocket.send(capitalizedSentence.encode())

  - connectionSocket.close()

- Transport layer only on the host device (operating system)

  - provides logical communication between application processes running on different hosts

  - "a process to process communication"

- TCP: Transmission Control protocol

  - reliable, in order delivery

    - "no message boundaries"

  - congestion control

  - flow control

    - sender will not overwhelm receiver

  - connection setup

  - socket identified by 4-tuple

    - source IP address

    - source port number

    - dest IP address

    - dest port number

  - one sender one receiver

  - bi-directional  data flow in same connection

  - MSS: Maximum segment size

  - cumulative ACKs

  - contains sequence number

| source port # |  |  |  |  |  |  |  | dest port # |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sequence number |  |  |  |  |  |  |  |  |
| acknowledgement number (what # sender is expecting to hear back from receiver) |  |  |  |  |  |  |  |  |
| head len | not used | U | A | P | R | S | F | receive window (how much buffer receiver has) |
| checksum |  |  |  |  |  |  |  | Urg data pointer |
| options (variable length) |  |  |  |  |  |  |  |  |
| Application data (variable length) |  |  |  |  |  |  |  |  |

- UDP:

  - unreliable, unordered delivery

  - no frills extension of "best-effort" IP

    - Segments may be lost or delivered out of order

  - No handshake performed

    - Can function when network service is compromised

  - when performing demultiplexing uses dest port number only

  - checksum

    - detect errors in transmitted segment

    - Adds two 16-bit integers and checks the sum with the checksum

    - 1’s complement of the sum

    - not perfect

  - connectionless

    - no handshaking between UDP sender and receiver

      - faster by one RTT

    - no connection state at sender, receiver

      - saves memory

    - smaller segment size

    - no congestion control

  - uses

    - streaming multimedia apps (loss tolerant)

    - DNS

    - SNMP

    - HTTP/3

      - add needed reliability & congestion control at application layer

| source port # | destination port # |
| --- | --- |
| other header fields |  |
| application data (payload) |  |

- multiplexing

  - happens at all layers

  - sender

  - handle data from multiple sockets, add transport header

- demultiplexing

  - use header info to deliver received segments to correct socket

- when creating socket, must specify host local port #:

  - mySocket = socket(AF_INET,SOCK_DGRAM)

  - mySocket.bind(myaddr,9157);

- Reliable Data Transfer

  - receiver sends a message back

  - sending process:

    - rdt_send(data)

      - pushes data to transport layer

    - packet = make_packet(data)

    - udt_send(packet)

      - called by transport layer and transfers segment to the network layer

  - receiving process

    - rdt_rcv(packet)

      - called by transport layer on the receiving side

    - extract(packet,data)

    - deliver_data(data)

  - Separate FSMs for sender and receiver

    - Stop and Wait protocol

      - receiver sends back acknowledgements (ACK)

      - can send back negative acknowledgements (NAK) if checksum doesn't match

        - sender retransmits

    - rdt 2.0 has a fatal flaw

      - ack/nak could be corrupted

      - sender will resend

      - receiver needs to identify potential duplicate packets

        - receiver discards duplicate

    - rdt 2.1

      - make_pkt(sequence#,data,checksum)

      - make_pkt(ACK, chksum)

- Selective repeat

  - receiver individually acknowledges all correctly received packets

    - buffer packets as needed for eventual inorder deliver to upper layer

  - sender will maintain a timer for each unACKed packet

    - determines delay

    - only has timer for one unACKed packet

    - sequence number needs to be > 2x the window size

  - Sequence number is byte-stream number of first data byte in segment

- TCP timeout value

  - Needs to  be longer than RTT

    - This value isn’t known so we estimate it

    - EstimatedRTT = (1-α)EstimatedRTT + α*SampleRTT

      - Exponential weighted moving average

      - Influence of a past sample decreases exponentially fast

      - Typical value:  α=0.125

      - We need a safe margin for RTT’s above the estimate

    - TimeoutInterval = EstimatedRTT + 4*DevRTT

              - Safety Margin

    - DevRTT = (1-ꞵ)*DevRTT + ꞵ*|SampleRTT-EstimatedRTT|

- TCP uses a hybrid of timeout and selective repeat

- Receive buffers packets

- If a packet has sequence number of 92, and it has 8 bytes,it has sequence numbers 92-99 included, and the receiver will ACK 100.

- Receiver can only send a cumulative  acknowledgement if it has received everything prior

  - This covers lost ACKs in between when returning to the sender

- TCP fast transmit

- 3-way Handshake

  - client state

    - clientSocket = socket(AF_INET,SOCK_STREAM)

    - clientSocket.connect((serverName,serverPort))

    - Listen, SYNSENT, ESTAB

  - Server state

    - serverSocket=socket(AF_INET,SOCK_STREAM)

    - serverSocket.bind(('',serverPort))

    - serverSocket.listen(1)

    - connectionSocket,addr = serverSocket.accept()

    - Listen, SYN RCVD, ESTAB

- Connection close

  - send TCP segment with FIN bit = 1

  - ACK and FIN bit sent back as 1

- don't need to memorize rdt2.2 for exam just understand and read it

- Congestion Control

  - Long delays

  - Packet loss due to buffer overflow

  - Original data: λin and throughput: λout

  - Host A and Host B sending through the same link to two separate servers

  - Q: what happens as arrival rate λin approaches R/2

  - A: maximum throughput λout is R/2

    - As packets are lost, due to handling duplicates, λout won’t reach R/2

  - Delay λin will exponentially increase as it goes to R/2

  - As packet loss occurs, we will have λin and λ’in of retransmitted data

  - Given multiple routers throughput can collapse to 0 if another host is filling up the buffer first

  - Network-assisted congestion control: routers provide direct feedback to hosts passing through congested router

    - used by google

  - TCP congestion control (AIMD)

    - implemented by OS

    - senders can increase sending rate until packet loss occurs, then decrease sending rate

      - Cut in half on loss detected by triple duplicate ACK (TCP Reno)

      - Cut to 1 MSS (maximum segment size) when loss detected by

timeout (TCP Tahoe)

    - TCP rate= cwnd/RTT

      - cwnd = congestion window

    - LastByteSent-LastByteACKed < cwnd

    - initial cwnd = 1MSS

    - double cwnd every RTT

      - once it hits ssthresh it switches to linear growth

        - on loss, ssthresh is set to ½ of cwnd

    - TCP Cubic uses polynomial increase instead of linear

      - Better than the linear increase

    - Delay-based TCP congestion control

      - Measure the RTT of every packet you send

        - Will start small (RTTmin)

        - With RTTmin calculate uncongested throughput

          - cwnd/RTTmin

        - Measured throughput = (#bytes sent)/RTTmeasured

          - if close to RTTmin increase linearly

          - if far from RTTmin decrease linearly

    - Explicit congestion notification

      - ECN flag on IP packet will show congestion

      - client sends ECE=1 in ACK segment to let the server know there is congestion

    - additive increase and multiplicative decrease gives TCP fairness in idealized assumptions; same RTT< fixed number of session in congestion

      - connection 1 throughput R = connection 2 throughput R

      - if bottleneck is in the access network (ethernet vs cable) it won't be "fair", bottleneck link must be shared

  - UDP doesn't support congestion control

    - multimedia apps do not use TCP

      - audio/video/gaming

  - QUIC: quick UDP internet connections

    - increase performance of HTTP

    - deployed on Chrome, youtube app

    - implemented on application layer instead of transport layer

    - TCP has 1 handshake performed, then TLS handshake performed

    - QUIC has one handshake done

    - HTTP/2 uses QUIC

    - QUIC handshake can be sent stream out of order so it doesn't have to wait for a retransmission to send the next steps, sends and retransmit

### Network Layer

- finding the optimal path from the router to the receiver

  - transfers datagram from sender to receiver

    - sender encapsulates segments into datagrams,passes to link layer

    - receiver delivers segments to transport layer protocol

  - "service to deliver"

- network layer protocol in every internet device: hosts, routers

  - routers:

    - examines header fields in all IP datagrams passing through it

- Forwarding

  - move packets from a routers input link to output link

  - determine route taken by packets from source to destination

    - coordinate with all routers and find the optimal route

- Data plane:

  - based on the header in the datagram it will decide where to forward to

- Control plane:

  - for finding an optimal route, two types of routing algorithms

    - most use traditional routing algorithms

    - some remote servers use software-defined networking(SDN)

      - runs routing algorithm on a server

- Forwarding table shows datagram header and output link(port)

  - routing algorithms populate the forwarding table

- Network layer has no guarantees

- IP datagram

  - Link layer encapsulated IP datagram

  - contains the header and the payload

    - header has the source and destination address

- Destination based forwarding

- generalized forwarding

  - based on any set of header field values rather than just the destination IP

- We use longest prefix matching to find the link interface

- Review hash table

- how much buffering

- (RTT*C)/√N

  - N is the number of flows

  - C is the link capacity

- IP protocol

  - IP Datagram format

    - TTL: time to live

      - each time passes through router decreased by one so a lost packet dies

    - header checksum

      - recalculated due to changing TTL

  - IP fragmentation

    - FDDI (Fiber) supports more bytes than ethernet

    - large IP datagram is fragmented within net

      - reassembled at destination

      - all fragments carry the same identifier in the ident field

      - IP header has instructions on reassembly

        - offset = bit total/8

  - IP addressing

    - 32 bit

    - 256.256.256.256

    - Subnet

      - devices that can connect with each other without passing through a router

      - higher bits denote a subnet

      - subnet mask: /24, ex: 223.1.2./24

  - How does an ISP get a block of address

    - ICANN

      - allocates IP addresses to the five regional internet registries (RIR) around the world

      - The RIRs then allocate smaller IP address blocks to ISPs

      - allocated last chunk of IPv4 addresses to RRs in 2011

        - NAT helps IPv4 address space exhaustion

          - all devices in a local network share just one IPv4 address as far as the outside world is concerned

    - 4 billion IP addresses, broken up into Class A, B, C (outdated)

      - Class A network supports 16,777,214 hosts

      - Class B network supports 65,534 hosts

      - Class C supports 254 hosts

        - .0 is reserved for the subnet, .255 is reserved for broadcasting

    - Classless Inter Domain Routing (CIDR)

      - allocates a contiguous class C address

        - assume we assign numbers 192.4.16-192.4.31

        - The first 20 bits are the same so we write out

          - 192.4.16/20

          - 192.4.16.0/24 would denote a subnet

          - 4 next bits used to denote department

          - last 8 are the 254 hosts

        - /24, /25, /26

- DHCP server has name and IP address of local DNS server that it gives to hosts

  - DHCPS uses UDP

  - gives the host the IP address of the router

- IPv6

  - 128 bit instead of 32 bit

  - Enable different network layer treatment of flows

    - Better quality of service

    - Some security associated with it

    - No checksum, no fragmentation

  - Tunneling and encapsulation

    - Used when there's not a straightforward IPv6 connection and uses some IPv4 routers in between

    - Uses IPv6/IPv4 routers

    - Encapsulates datagram to IPv4original datagram inside a IPv4 datagram

      - the IPv6/IPv4 router will become the source and the next IPv6/IPv4 router will become the destination

- generalized forwarding

  - match:pattern values in packet header fields

  - actions: for matched packet: drop, forward, modify, encapsulate and forward

  - priority: disambiguate  overlapping patterns

  - counters: #of bytes and packets

- OpenFlow abstraction

  - match+action:

    - ex: 	match			action

      - ingress port = 1

      - IP Src =10.3.*.*	forward(3)  #forwards over port 3

      - IP Dst =10.2.*.*

  - can do things like specify route

- Middleboxes

  - any intermediary box performing functions apart from routing

  - NAT,  Firewalls, IDS, load balancers, caches

    - load balancers split traffic to different servers

  - movetowards "whitebox" hardware implementing open API

  - Network functions virtualization

  - SDN

- Architectural Principles of the Internet

  - simple connectivity

  - IP protocol: (thin waist of IP hourglass, network layer is the simplest layer)

  - intelligence, complexity at network edge

- now the IP hourglass has "love handles" because of middleboxes

- Two approaches to structuring network control plane

  - Per-router control plane

    - individual routing algorithm components in each and every router

  - Software-Defined Networking control plane (SDN)

    - server does routing algorithm

- Routing protocol goal

  - determine good paths from sending hosts to receiving host

    - "good" = least cost/fastest/least congested

  - routing algorithm is a graph based algorithm

- D(v)= min(D(v),D(w),+cw,v)

- Dijkstra’s algorithm ex (network layer 5-16)

| Step | N’ | v | w | x | y | z |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | u | 7,u | 3,u | 5,u | inf | inf |
| 1 | uw | 6,w |  | 5,u | 11,w | inf |
| 2 | uwx | 6,w |  |  | 11,w | 14,x |
| 3 | uwxv |  |  |  | 10,v | 14,x |
| 4 | uwxvy |  |  |  |  | 12,y |
| 5 | uwxvyz |  |  |  |  |  |

- Time complexity:

  - N nodes gives time complexity O(n2) when using adjacency matrix

  - When using adjacency matrix with priority queue we get O(nlogn)

  - Network operator

- We used bandwidth and congestion level to reroute instead of just using traffic volume

- Distance vector algorithm

  - Bellman Ford Equation:

  - Dx(y)=minv{cx,v+Dv(y), cx,y+Dy(y)}

    - distance of y from x, v being neighbors

    - min of all neighbors

  - Difference is that at the begging the Distance vector based algorithm, each node doesn't know the bigℼcture only it's neighbors distances

    - each node will send info to its neighbors, neighbors forward to neighbors..

      - sends distance vector

      - when x receives new DV estimate from any neighbor it updates its own DV using BF equation

      - the estimate converges to the real value

    - wait → recompute →  notify →  repeat ↻

      - eventually algorithm will stop updating

      - Nodes detect local link cost change and notify neighbors

    - We use poisoned reverse technique as a solution to infinitely updating neighbors

- Comparison of Link State and Distance Vector algorithms

  - Message complexity:

    - LS: n routers, O(n2) messages sent

    - DV: exchange between neighbors; varies

  - Speed of convergence

    - LS: O(n2)

    - DV: convergence time varies with count to infinity problem

  - Robustness: what happens if the router malfunctions or is compromised?

    - LS: router can advertise incorrect link cost

      - Each router computes only its own table

    - DV: router can advertise incorrect path

      - Each router’s table used by others: error propagates through net

- intra-ISP routing: OSPF

  - We have “autonomous systems”/”domains”

    - Local networks such as UD’s network or Spectrum in Dayton

  - intra-AS aka “intra-domain”

    - All routers in AS must run same intra-domain protocol

    - Routers in different AS can run different intra-domain routing protocols

    - Gateway router: at “edge” of its own AS, gets packets and sends packets from routers in other AS’es

    - policy<performance

  - inter-AS

    - Routing among AS’es

    - Gateway perform inter-domain routing also

    - performance > policy

  - OSPF routing algorithm

    - Two-level hierarchy: local area and backbone

      - LS advertisements flooded only in one area

      - Each node has detailed area topology

      - Only knows direction to reach other destinations

- Routing along

  - BGP (Border Gateway Protocol)

    - “Glue that holds the internet together”

    - used to populate routing table

    - Tells internet “i am here, here is who i can reach, and how”

    - BGP provides

      - eBGP: (external) obtain subnet reachability info from neighbor AS

      - iBGP: (internal) propagate reachability info to all routers in AS

    - BGP session: two BGP routers exchange BGP messages over semi-permanent TCP connection

      - Propagates a path

    - BGP advertised route: prefix + attributes

      - prefix: destination ex: 256.10/16

      - Attributes:

        - AS-PATH: list of ASes through which prefix advertised

        - NEXT-HOP: indicates specific internal-AS router to next-hop AS

    - BGP messages (using TCP)

      - OPEN: open TCP connection

      - UPDATE: advertise new path

      - KEEPALIVE: keeps connection alive, also ACKs OPEN request

      - NOTIFICATION: reports errors and used to close connections

    - Benefit of less traffic and outers in n in O(n2)8

      - doesn't have to have every entry for every router, just the eBGP routers

    - BGP will advertise to all neighbors but some won't accept due to not wanting to handle other ISP's traffic

      - ISP customers don't want to be middlemen

- SDN control plane

  - server can check for errors instead of having them propagate through the network

  - flow based generalized forwarding

  - table based forwarding (OpenFlow API) allows "programming" routers

    - easier to run on one machine than every machine, then distribute

  - old routers don't support SDN

  - uses Dijkstra or Bellman Ford

  - you can split traffic

- ICMP: Internet Control  Message Protocol

  - used by hosts and routers to communicate network-level info

    - ex:ℼng to check if destination is reachable

    - network-layer "above" IP:

    - Traceroute:

      - source sends sets of UDP segments to destination

        - 1st set has TTL=1, 2nd set has TTL=2,etc.

        - router will send back ICMP message saying TTL expired

        - this can record RTTs

- Link Layer: introduction

  - terminology

    - hosts and routers: nodes

    - communication channels that connect adjacent nodes along communication path: links

      - wired - DSL, fiber optics, television line

      - wireless - 4G/5G, wifi

      - LAN

    - layer-2 packet: frame, encapsulates IPdatagram

    - Link layer has a responsibility of transferring datagram from one node to physically adjacent node over a link

  - Cyclic Redundancy Check (CRC)

    - More powerful error-detection coding

      - D: data bits

      - G: bit pattern of r+1 bits

        - both nodes know the bit pattern

        - formula for bit pattern: <D,R> = D*2r XOR R

        - we want D*2r XOR R = nG so that the remainder is 0

        - if we divide D*2r by G we want remainder R to satisfy

          - R=remainder[(D*2r)/g

- Multiple access protocols

  - collisions in multiple access protocols

    - protocol determines when node can transmit

    - communication about channel sharing must use the channel itself!

    - no out of band channel for coordination

    - FDMA

      - channel separated into different frequency bands

        - pro: no time slots

        - con: higher frequencies have lower bandwidth

    - random access protocol

      - transmit at full data rate

  - Slotted ALOHA

    - first wireless protocol - 1970 radio transmission in hawaii islands

    - decentralized

    - assumptions:

      - all frames same size

      - time divided into equal size slots

      - nodes start to transmit only slot beginning

      - nodes are synched

      - if 2 or more nodes transmit in slot, all nodes detect collision

    - when node obtains fresh frame, transmits in next slot

      - if no collision: node can send new frame in next slot

      - if collision: node retransmits frame in each subsequent slot with probability p until success

    - prob that given node has success in a slot = p(1-p)N-1

    - prob that any node has a success = Np(1-p)N-1

    - max efficiency: find p* that maximizes Np(1-p)N-1

    - for many nodes, take limit of Np*(1-p*)N-1 as N goes to infinity, gives max efficiency = 1/e = .37

  - CSMA (Carrier sense multiple access)

    - if channel sensed idle: transmit entire frame

    - if channel sensed busy: defer transmission

    - "don't interrupt others!"

  - CSMA/CD: w/ collision detection

    - collisions detected w/ in short time

      - collisions when delay stops one node from knowing another is transmitting data

    - transmissions aborted

    - detection difficult with wireless

      - used with ethernet

  - CSMA/CD Ethernet algorithm

    - after mth collision NIC chooses random K from 0,1,2,…,2m-1, waits K*512 bits

    - efficiency=1/(1+5tprop/ttrans)

      - goes to 1 as tprop goes to 0 or as ttrans goes to infinity

- A switch is a link layer device

  - connects multiple ethernet cables

  - switch doesn't have the MAC address

    - host is unaware of the switch b/c this (transparent)

    - no configure required

  - self learning

    - has a switch table

      - MAC address of host, interface to reach host, TTL

      - similar to routing table

      - if switch doesn't find the destination it will flood to all ports

        - will add destination to table

- VLAN: Virtual Local Area Network

  - All users use one switch where the ports are split into different VLANs

  - You can use software to change the # of ports for each VLAN

    - solves unused port issue separate switches has

    - helps isolate traffic so broadcasts aren't overset

  - We use trunk ports for scalability

    - forwards frames across VLAN switches

- Layer 3 switch

  - built in router

- ARP

  - ARP query broadcast to find MAC address of neighboring routers

  - you know the IP address of servers but not MAC address of each port.

- Wifi:

  - Collision avoidance

    - You don’t need collision avoidance, adds delay

  - The sender acts as the original host in frame addressing

  - Sequence control field is for reliable data transfer

    - Wifi is noisy so it needs some reliability

    - As long as you're connecting to same access point your IP doesn't change

- 4G:

  - 64 bit sim identification

  - ~7k range per tower

  - Your IP is not a public IP while on 4G, its private

  - You can track location with cellular network connection

- remember TCP rate = cwnd/RTT in bytes/sec