Subnet A has 230 hosts, so it will need at least 232 addresses (including the subnet ID and broadcast address). The least number of bits that satisfy this is 8 bits. Knowing that, we take the prior subnet and add 256, the result of which is 172.26.254.0/24

The broadcast address of subnet A (172.26.254.0/24) is 172.26.254.255, because it is the last address in the IP range.

The first usable host IP address of subnet A (172.26.254.0/24) is 172.26.254.1, found by adding 1 to the subnet address (i.e., subnet ID).

The last usable host IP address of subnet A (172.26.254.0/24) is 172.26.254.254, found by subtracting 1 from the broadcast address (172.26.254.255).

Similar to the prior subnet, subnet B has 118 hosts, so it will need at least 120 addresses (including the subnet ID and broadcast address). The least number of bits that satisfy this is 7 bits. Knowing that, we take the prior subnet and add 128, the result of which is 172.26.255.0/25

The broadcast address of subnet B (172.26.255.0/25) is 172.26.255.127, because it is the last address in the IP range.

The first usable host IP address of subnet B (172.26.255.0/25) is 172.26.255.1, found by adding 1 to the subnet address.

The last usable host IP address of subnet B (172.26.255.0/25) is 172.26.255.126, found by subtracting 1 from the broadcast address (172.26.255.127).

Similar to the prior two subnets, subnet C has 41 hosts, so it will need at least 43 addresses (including the subnet ID and broadcast address). The least number of bits that satisfy this is 6 bits. Knowing that, we take the prior subnet and add 64, the result of which is 172.26.255.128/26

The broadcast address of subnet C (172.26.255.128/26) is 172.26.255.191, because it is the last address in the IP range.

The first usable host IP address of subnet C (172.26.255.128/26) is 172.26.255.129, found by adding 1 to the subnet address.
The last usable host IP address of subnet C (172.26.255.128/26) is 172.26.255.190, found by subtracting 1 from the broadcast address (172.26.255.191).

### Step-by-step assignment:

#### Base network: 172.26.254.0/23

- Range: 172.26.254.0 – 172.26.255.255

- 512 addresses total

### 1. Subnet A – 230 hosts

- Needs: 230 + 2 → 232 → /24 (256 IPs)

- Assign: 172.26.254.0/24

- Range: 172.26.254.1 – 172.26.254.254

### 2. Subnet B – 118 hosts

- Needs: 120 → /25 (128 IPs)

- Next block starts at: 172.26.255.0/25

- Range: 172.26.255.1 – 172.26.255.126

### 3. Subnet C – 41 hosts

- Needs: 43 → /26 (64 IPs)

- Next block: 172.26.255.128/26

- Range: 172.26.255.129 – 172.26.255.190

| Subnet | CIDR Notation | Broadcast | Host Range | Size |
| --- | --- | --- | --- | --- |
| A | 172.26.254.0/24 | 172.26.254.255 | 172.26.254.1 – 172.26.254.254 | 256 |
| B | 172.26.255.0/25 | 172.26.255.127 | 172.26.255.1 – 172.26.255.126 | 128 |
| C | 172.26.255.128/26 | 172.26.255.191 | 172.26.255.129 – 172.26.255.190 | 64 |